PROBLEM 4 — BILL OF MATERIALS (BOM) FOR AN RC AIRCRAFT

Electric, Single-Motor Fixed-Wing Aircraft | Marks: 25 (+5 Bonus)

1. Aircraft Brief Requirements

| Parameter | Requirement Specification |
| --- | --- |
| Wingspan | 1.0 – 1.2 m |
| Configuration | High wing / Conventional fixed-wing configuration |
| Propulsion | Single electric brushless motor |
| Control | 4-Channel Wireless Remote Control (Throttle, Ailerons, Elevator, Rudder) |
| Structure | Lightweight, crash-resistant, and easily repairable (Foam / Balsa / Carbon Reinforcement) |
| Takeoff Mode | Hand launch or Smooth Ground Takeoff (Tricycle / Taildragger gear) |

2. Part A & B — Component Identification & Trade-Off Comparison

| Component | Recommendation 1 | Recommendation 2 | Key Specifications / Notes | Qty | Est. Cost (₹) |
| --- | --- | --- | --- | --- | --- |
| Motor | SunnySky X2212 1400KV | A2212 1000KV Brushless | 1400KV, 20A max, 3S compatible, 2-3S LiPo | 1 | 1,200 |
| ESC | Flycolor 30A ESC | Yellow Jacket 30A ESC | 30A continuous, 2A/5V BEC integrated | 1 | 650 |
| Propeller | 10x4.5 SF Propeller (2-pack) | 9x6E Electric Propeller | 10x4.5 2-Blade Slow Flyer, 3.17mm adapter | 1 pair | 250 |
| Battery | Orange 3S 2200mAh 30C | Bonka 3S 1500mAh 45C | 11.1V 3S LiPo, 2200mAh, 30C discharge, XT60 | 1 | 1,450 |
| Airframe | Expanded Polypropylene (EPP) | Corrugated Plastic (Coroplast) | 1.1m wingspan high-wing trainer, repairable | 1 kit | 1,800 |
| Reinforcement | Carbon Fiber Rod 3mm | Fiberglass Tape 25mm | 3mm OD x 1m carbon spar for wing rigidity | 2 | 300 |
| Servos | TowerPro SG90 9g Servo | EMAX ES08A II 9g Servo | 9g micro servo, 1.8 kg·cm torque, plastic gear | 4 | 600 |
| Transmitter & Rec. | FlySky FS-i6 + IA6B | Radiomaster Pocket CC2500 | 6-Channel 2.4GHz AFHDS 2A, PWM/PPM Rec. | 1 | 4,200 |
| Landing Gear | Aluminum Main + Tail Wheel | Carbon Fiber Fixed Gear | Lightweight pre-bent wire/aluminum + 2" wheels | 1 set | 350 |
| Hardware & Glue | Servo Horns, Pushrods, CA | Hot Glue + Pushrod Connectors | 1.2mm pushrods, stopper, CA glue, XT60 plug | 1 set | 400 |

3. Part C — Engineering Selection & Technical Justification

1. Motor–Propeller Match & Thrust Verification: A 1400KV motor with a 10x4.5 prop on 3S LiPo produces ~850g static thrust at 16A current draw. Thrust is verified using a digital thrust-stand setup measuring static force against current draw.

2. ESC Rating & Electrical Margin: Maximum continuous current draw from motor is ~16A–18A. Selecting a 30A ESC provides a safe ~40% electrical margin, preventing thermal shutdown and ensuring longevity.

3. Battery Selection & Trade-Offs: A 3S 2200mAh 30C LiPo offers an optimal balance between weight (185g) and flight time (~10–12 minutes). Higher capacity adds excessive mass without proportional runtime benefit.

4. Servos & Control Surfaces: 4 micro servos (9g each) assigned to: Aileron Left, Aileron Right (dual-servo wing setup), Elevator (pitch control), and Rudder (yaw / ground steering control).

5. Structural Material Selection: EPP foam is chosen for the airframe due to high impact resistance and simple field repairability using CA glue or hot glue. Carbon fiber spars provide essential flexural rigidity.

6. Electrical & Mechanical Compatibility: ESC BEC delivers 5V regulated power to the RX receiver and 4 servos via standard 3-pin DuPont headers. 2.4GHz AFHDS 2A protocol ensures robust frequency-hopping against interference.

4. Part D — Budget Analysis & Configuration Trade-Offs

1. Total Estimated Build Cost: Estimated complete build cost for the chosen setup is approximately ₹11,200 (including transmitter/receiver transmitter kit). Component-only build cost without TX/RX is ~₹7,000.

2. Safe Cost-Saving Opportunities: Reusing an existing transmitter/receiver saves ₹4,200. Utilizing EPP/Coroplast sheet DIY build instead of a pre-molded kit saves ~₹800 safely without sacrificing structural integrity.

3. Budget vs. Performance Configurations: Budget Configuration (₹8,500 total): A2212 motor, 30A non-branded ESC, 3S 1500mAh battery, Coroplast frame.
Performance Configuration (₹13,500 total): SunnySky X2212 motor, Flycolor 30A ESC, EPP pre-molded frame, Digital metal-gear servos.

4. Selected Competition Configuration & Defense: The standard balanced performance setup (₹11,200 complete) is selected. It guarantees high thrust-to-weight ratio, structural repairability, and reliable frequency-hopping link crucial for competition environment.

5. Bonus (+5 Marks) — Takeoff Mass & Propulsion Assessment

| Subsystem Component | Mass Estimate (g) |
| --- | --- |
| Airframe (EPP + Carbon Rods + Hardware) | 280 g |
| Propulsion (Motor + ESC + Propeller) | 110 g |
| Battery (3S 2200mAh LiPo) | 185 g |
| Electronics (4x Servos + Receiver + Wiring) | 65 g |
| Total All-Up Mass (AUW) | 640 g |

Propulsion Assessment: With an estimated Total Takeoff Mass (AUW) of 640g and available static thrust of 850g (from 1400KV motor on 10x4.5 prop), the resulting Thrust-to-Weight Ratio is 850g / 640g = 1.33:1. A ratio > 1.0 ensures effortless hand launch, rapid rate of climb, and agile recovery performance under competition conditions.